IoT and Smart Devices Assignments

104240618 – Lay Võ Đức

This file includes the assignments of lectures “3-sensors-and-actuators” and “4-connect-internet” inside the folder “1-getting-started”

Assignment of “3-sensors-and-actuators”:

Requirement: This lesson covered sensors and actuators. Research and describe one sensor and one actuator that can be used with an IoT dev kit, including:

- What it does

- The electronics/hardware used inside

- Is it analog or digital

- What the units and range of inputs or measurements is

Sensor: Pyranometer

- It measures incoming solar radiation on a plane surface.

- Components: Thermopile (thermocouples), black absorber plate, glass domes for spectral response and cosine correction.

- It’s primarily analog voltage.

- Units and range: W/m², typically 0 to 2000 W/m² (up to 4000 W/m²).

Actuators: Servo Motor

- It controls position, speed, and torque in actuators for movement.

- Components: DC motor, reduction gearbox, potentiometer/encoder for feedback, PWM control circuit.

- It’s digital

- Units and rage: Degrees (0°-180° or 270°), torque in kg·cm (1-50 depending on model).

# Assignment of “4-connect-internet”

Requirement: This lesson covered MQTT as a communication protocol. There are others, including AMQP and HTTP/HTTPS.

Research these both and compare/contract them with MQTT. Think about power usage, security, and message persistence if connections are lost.

- Comparison between MQTT and AMQP

| Criteria | MQTT | AMQP |
| --- | --- | --- |
| Power Usage | It’s lightweight for low power devices. It uses tiny headers and a function to maintain connections without constant reconnecting -> Reduce the power consumption. | It’s higher than MQTT. It’s more complex than MQTT so it involves more data processing and processing power. |
| Security | Connections can be public and open. It can also be secured using passwords, usernames, certificates or encryption. | It is a standard communication protocol used to connect to cloud servers. So although it’s like MQTT, it requires authenticated methods. |
| Message Persistence | It relies on QoS levels - 3 levels. It uses a retained flag to store the last message on a topic for new subscribers. | Messages are stored in queues to ensure delivery even if a client is offline temporarily. |

- Comparison between HTTP/HTTPs and MQTT

| Criteria | MQTT | HTTP/HTTPs |
| --- | --- | --- |
| Power Usage | It’s lightweight for low power devices. It uses tiny headers and a function to maintain connections without constant reconnecting -> Reduce the power consumption. | HTTP costs more energy due to large, text-based headers. The closing and opening of TCP connections cost significant energy. |
| Security | Connections can be public and open. It can also be secured using passwords, usernames, certificates or encryption. | It uses HTTPS (HTTP + TLS/SSL) -> Highly secure. The encryption layers add “weight” to the data packers -> Enhance the secured method. |
| Message Persistence | It relies on QoS levels - 3 levels to ensure the transmission even if the connection is unstable. It uses a retained flag to store the last message on a topic for new subscribers. | HTTP is generally stateless. If the connection is lost during the transmission, the data is usually lost, unless the application is manually programmed to retry the entire request. |